COOKIE

A Teaching Guide to the Robot Code

Team 1555 - FRC Robot Programming
Command-Based Java / WPILib

| Constants | Subsystems | Commands | LED / LEDs |
| --- | --- | --- | --- |

What each colored section means

1.  How the Robot is Organized

Cookie is a simple demo robot: a tank-drive chassis with a bag-motor drivetrain, an arm that lifts, an intake roller, and a strip of LEDs. It is written in Java using WPILib's command-based framework, the standard way FRC robots are programmed.

The whole project is split into three kinds of files, and it helps to think of them as three different jobs:

Constants.java - the robot's address book. It holds every port number and speed, so wiring and tuning are in ONE place.

Subsystems - the robot's body parts. Each file controls one physical mechanism (drive, arm, intake, LEDs).

Commands + RobotContainer - the robot's brain. They decide WHEN body parts move and how buttons are wired to actions.

A subsystem never decides when to run. A command tells it to. RobotContainer is where buttons on the controller get connected to commands. This separation is the whole point of command-based programming - each piece has exactly one job.

2.  Constants.java - The Robot's Address Book

Every number that describes the robot lives here. If you need to change a port or a speed, you change it HERE and never hunt through other files. There is no code logic here - just named numbers.

public static class DriveConstants {

public static final int kLeftMotorPwmPort = 0;

public static final int kRightMotorPwmPort = 1;

public static final double kDefaultDriveSpeed = 0.3;

}

The drive has ONE PWM port per side. Each port is Y-split to two Victor controllers (one per bag motor).

kDefaultDriveSpeed = 0.3 means the robot starts at 30% power so beginners can't send it across the room.

public static class ArmConstants {

public static final int kArmMotorPwmPort = 2;

public static final int kTopLimitSwitchDioPort = 0;

public static final int kBottomLimitSwitchDioPort = 1;

public static final double kArmUpSpeed = 0.3;

public static final double kArmDownSpeed = -0.15;

public static final double kArmDownRampStart = 0.05;

public static final double kArmDownRampTime = 0.6;

public static final double kArmHoldSpeed = 0.08;

}

Port 2 is the arm motor. DIO 0 and DIO 1 are the two limit switches that tell the arm it reached the top or bottom.

The down speed is negative (motor spins the other way) and smaller than up - the arm has gravity helping it, so it doesn't need as much power.

The RAMP values make the arm start slow and speed up, so it doesn't slam down. The HOLD speed keeps it up at the top switch without burning the motor.

public static class IntakeConstants {

public static final int kIntakeMotorPwmPort = 7;

public static final double kIntakeSpeed = 0.7;

}

Port 7 runs the intake roller. Same speed is used forward and reverse; the minus sign flips the direction.

public static class BlinkinConstants {

public static final int kBlinkinPwmPort = 5;

public static final double SOLID_BLUE = 0.87;

public static final double SOLID_RED  = 0.61;

public static final double SOLID_GREEN = 0.77;

public static final double CONFETTI = -0.87;

public static final double LARSON_SCANNER = -0.51;

...

public static final double DRIVE_FORWARD = LARSON_SCANNER;

public static final double SHOOTING = SOLID_RED;

public static final double INTAKING = SOLID_GREEN;

}

PWM port 5 feeds the REV Blinkin LED driver. Each number is one of the ~100 patterns built into the Blinkin.

The named constants (like SOLID_GREEN) make the code readable. Instead of setPattern(0.77), the code says setPattern(SOLID_GREEN).

3.  Robot.java - The Heartbeat

Robot.java is the WPILib base class. It runs the Scheduler every 20 milliseconds. The Scheduler is the engine that makes command-based programming work - it checks which buttons are pressed, starts and stops commands, and calls each subsystem's periodic() method. As a rule, you almost never edit this file.

public void robotPeriodic() {

CommandScheduler.getInstance().run();

SmartDashboard.putNumber("Robot/Battery Voltage",

RobotController.getBatteryVoltage());

SmartDashboard.putBoolean("Robot/Brownout",

RobotController.isBrownedOut());

}

CommandScheduler.getInstance().run() - the heartbeat. Everything ticks because of this one line.

Battery voltage and brownout go to the dashboard so the driver can see the robot's health while driving.

4.  RobotContainer.java - Wiring Buttons to Actions

RobotContainer creates all the subsystems, then connects controller buttons to commands. Think of it as the wiring diagram for the brain. It is where the driver's inputs become robot behavior.

private final DriveTrainSubsystem m_drivetrain = new DriveTrainSubsystem();

private final ShooterSubsystem   m_shooter   = new ShooterSubsystem();

private final ArmSubsystem       m_arm       = new ArmSubsystem();

private final BlinkinSubsystem   m_blinkin   = new BlinkinSubsystem();

private final XboxController m_driveController = new XboxController(0);

Each subsystem is created exactly once and stored in a field (m_ prefix = member).

XboxController(0) is the controller in USB port 0 on the Driver Station.

m_drivetrain.setDefaultCommand(new RunCommand(

() -> m_drivetrain.driveTank(

MathUtil.applyDeadband(m_driveController.getLeftY(), 0.05),

MathUtil.applyDeadband(m_driveController.getRightY(), 0.05)),

m_drivetrain));

A default command runs automatically whenever nothing else needs the drivetrain - it's what makes the sticks drive the robot.

applyDeadband(x, 0.05) ignores tiny stick movements so the robot doesn't creep when the stick is slightly off-center.

new JoystickButton(m_driveController, XboxController.Button.kY.value)

.toggleOnTrue(new armUpCMD(m_arm));   // Y = arm up

new JoystickButton(m_driveController, XboxController.Button.kA.value)

.toggleOnTrue(new armDownCMD(m_arm)); // A = arm down

Y starts the arm-up command; pressing Y again stops it (toggleOnTrue).

new JoystickButton(m_driveController, XboxController.Button.kLeftBumper.value)

.onTrue(new RunCommand(() -> m_shooter.forwIntake(), m_shooter))

.onFalse(new RunCommand(() -> m_shooter.deadIntake(), m_shooter));

// Right bumper = backIntake (reverse)

onTrue while the bumper is held, onFalse when released - like holding a button on a game controller.

new POVButton(m_driveController, 0)

.onTrue(new InstantCommand(() -> m_drivetrain.setDriveSpeed(1), m_drivetrain));

// POV right = 0.75, down = 0.35, left = 0.5

The D-pad changes the drive speed scale on the fly - perfect for indoor events where full speed is scary.

5.  DriveTrainSubsystem.java - The Tank Drive

A tank drive has two motor groups: left and right. To go forward, both spin; to turn, they spin at different speeds. Each group is one PWM port Y-split to two Victor controllers.

public void driveTank(double Lspeed, double Rspeed) {

leftMotors.set(-Lspeed * currentDriveSpeed);

rightMotors.set(Rspeed * currentDriveSpeed);

}

The left side is NEGATED because the left and right gearboxes face opposite directions - the minus sign makes both sides go the same physical way when you push forward.

currentDriveSpeed is the throttle scale (0.3 default, changed by the D-pad). Multiply so 100% stick doesn't mean 100% full-power chaos.

public void periodic() {

SmartDashboard.putNumber("Drive/Left Output", leftMotors.get());

SmartDashboard.putNumber("Drive/Right Output", rightMotors.get());

}

periodic() runs every loop and sends live motor values to the dashboard - a great way to see if a motor is actually being commanded.

6.  ArmSubsystem.java + armUpCMD / armDownCMD - The Arm

The arm lifts up and down with one motor. Two limit switches (DIO 0 = top, DIO 1 = bottom) tell the code when the arm has reached the end of its travel so it doesn't keep driving and burn out.

public boolean topLimitSwitchPressed() {

return topLimSwitch.get();

}

IMPORTANT QUIRK: this method returns TRUE when the switch is NOT pressed and FALSE when it IS pressed. The wiring is 'normally closed'. Everywhere the code checks the arm, it uses !topLimitSwitchPressed() to mean 'at the top'.

// armUpCMD - drive up until the top switch, then HOLD

public void execute() {

m_armSubsystem.setArmSpeed(ArmConstants.kArmUpSpeed); // 0.3 up

}

public boolean isFinished() {

return !m_armSubsystem.topLimitSwitchPressed(); // stop at top

}

public void end(boolean interrupted) {

if (!interrupted && !m_armSubsystem.topLimitSwitchPressed()) {

m_armSubsystem.setArmSpeed(ArmConstants.kArmHoldSpeed); // 0.08 hold

} else {

m_armSubsystem.setArmSpeed(0);

}

}

isFinished() ends the command when the arm hits the top. end() then applies a small HOLD speed (0.08) so gravity doesn't pull it back down.

The hold is small on purpose: full power with the arm blocked = a burned motor. 8% keeps it up with almost no current.

// armDownCMD - start slow, ramp up, stop at the bottom switch

public void execute() {

double t = rampTimer.get();

double frac = Math.min(t / ArmConstants.kArmDownRampTime, 1.0);

double speed = ArmConstants.kArmDownSpeed * frac

+ ArmConstants.kArmDownRampStart * (1.0 - frac);

m_armSubsystem.setArmSpeed(speed);

}

public boolean isFinished() {

return !m_armSubsystem.botLimSwitchPressed();

}

The ramp blends from 5% up to full down speed over 0.6 seconds - smooth start, still fast overall. Gravity is already pulling, so it needs less motor power to go down.

7.  ShooterSubsystem.java - The Intake

One roller motor on PWM 7. It has three simple states: off, forward (output), reverse (intake).

public void forwIntake() { intake.set(IntakeConstants.kIntakeSpeed); }

public void backIntake() { intake.set(-IntakeConstants.kIntakeSpeed); }

public void deadIntake() { intake.set(0); }

Three tiny methods = the whole subsystem. The speed comes from Constants so it's tunable in one place.

getOutput() returns the current value so the LED logic can tell 'outputting' (+0.7) from 'intaking' (-0.7).

8.  BlinkinSubsystem.java - The LEDs

The Blinkin turns a PWM signal into LED patterns. The code treats it like a SPARK motor controller: setting a value like 0.87 makes it show a specific pattern. The numbers are defined as readable names in Constants.

public void setPattern(double pattern) {

currentPattern = pattern;

blinkin.set(pattern);

}

public void periodic() {

SmartDashboard.putNumber("LEDs/Pattern", currentPattern);

}

setPattern() is the ONLY way the code talks to the LEDs. Everything else decides WHICH pattern to call.

9.  updateLeds() - Making the Lights Tell a Story

This is the fun part. A single method runs every loop and picks ONE pattern based on what the robot is doing. Because only this method is allowed to touch the Blinkin, the lights can never fight each other.

private void updateLeds() {

// 1. Shooter wins: green = intaking, red = outputting

if (m_shooter.getOutput() < -0.05) { setPattern(INTAKING); return; }

if (m_shooter.getOutput() >  0.05) { setPattern(SHOOTING); return; }

// 2. Arm: violet when parked at top, confetti while moving

if (!m_arm.topLimitSwitchPressed()) { setPattern(ARM_AT_TOP); return; }

if (Math.abs(m_arm.getOutput()) > 0.05) { setPattern(ARM_MOVING); return; }

// 3. Drive: scanner forward, strobe reverse, flash turning

double ly = ...left stick...; double ry = ...right stick...;

if (both forward)  setPattern(DRIVE_FORWARD);

else if (both back) setPattern(DRIVE_BACKWARD);

else if (turning)  setPattern(DRIVE_TURN);

else               setPattern(IDLE); // solid blue

}

Priority order matters: shooter > arm > drive. If the intake is running while you drive, the intake color wins.

This is a state machine in disguise: the robot can only be in ONE state, and the LED matches it. Clean, simple, impossible for two patterns to clash.

10.  Building and Deploying

Getting your code onto the robot is two steps: build a jar, then copy it to the roboRIO.

Build: ./gradlew jar (in the project folder). This compiles Java into a runnable jar in build/libs/.

Deploy: copy the jar to /home/lvuser/ on the roboRIO (admin@10.15.55.2), then restart the program.

Restart: ssh in and run /usr/local/frc/bin/frcKillRobot.sh then /usr/local/frc/bin/frcRunRobot.sh (or just kill the java process - the rio restarts it).

The roboRIO runs NI Linux Real-Time. It has no systemctl - you restart the robot program with the frc scripts, and the DS must be restarted to clear an E-Stop latch.

11.  Troubleshooting Cheat Sheet

| Symptom | What to check |
| --- | --- |
| Motors blink red, don't move | Is the DS ENABLED? Spacebar = E-Stop. Reopen DS to clear. Controller showing 0 axes? Replug it. |
| Motor controller no signal | Check it's in a PWM port, not DIO. Check the port number matches Constants. |
| Arm won't stay up | Top switch polarity: code uses !topLimitSwitchPressed(). Check DIO 0 wiring. |
| Arm slams down | Lower kArmDownRampStart or lengthen kArmDownRampTime in Constants. |
| LEDs show wrong/static color | Check Blinkin is on PWM 5, strip on 5V select, data wire to DIN, and robot is ENABLED. |
| No 'Robot Code' on DS | Program crashed. Check rio log, restart program, verify jar deployed. |
| Loop overrun warnings | Flapping joystick. Use a good USB port; the joystick bindings can stall buttons.run(). |

Color guide: BLUE = drive/ports, GREEN = subsystems, RED = commands/inputs, ORANGE = LEDs, PURPLE = arm.